HAWASSA UNIVERSITY

INSTITUTE OF TECHNOLOGY

SCHOOL OF ELECTRICAL AND COMPUTER ENGINEERING

Peak Load Forecasting on a Case Study of Ethiopian Electricity Demand

By

Abebaw Gashaw (ID No. PGEngW/001/07)

A Thesis submitted to the school of Electrical and Computer Engineering in                    partial fulfillment of the requirement for the degree of

Master of Science

in

Power system and Energy Engineering

February, 2018

Hawassa, Ethiopia

HAWASSA UNIVERISTY

SCHOOL OF GRADUATE STUDIES

EXAMINERS’ APROVAL SHEET

As members of the board of examiners of the final Master’s Degree open defense, we certify that we have evaluated the thesis prepared by Abebaw Gashaw under the title ‘Peak Load Forecasting on a Case Study of Ethiopian Electric Demand’’ and recommend that it is accepted as fulfilling the thesis requirement for the degree of M.Sc. in Power System and Energy Engineering.

--------------------------------------           -------------------         ----------------------

Name of the chair person                   signature                             Date

--------------------------------------           -------------------         ----------------------

Name of internal examiner                 signature                             Date

--------------------------------------           -------------------         ----------------------

Name of external examiner                signature                             Date

--------------------------------------           -------------------         ----------------------

Principal Advisor                                signature                              Date

--------------------------------------           -------------------         ----------------------

Co  - Advisor                                      signature                              Date

Final approval and acceptance of the contingent upon the submission of the final copy thesis to the SG through the DGC/SGC of the candidate’s department/school /.

Thesis approved by:

----------------------------------              -----------------------          ---------------------SGS                                                   Signature                            Date

Declaration:

The under signed, declare that this thesis is my original work, has not been presented for a degree in thesis or any other university and all source of material used for the thesis have been fully acknowledged.

----------------------------------------------------                              ---------------------Name of student                                                                         signature

Place: Hawassa

Date of submission:  Feb, 2018

This thesis has been submitted for examination with my approval as university advisor.

--------------------------------------------                                     ------------------------Principal Advisor’s Name                                                          Signature

--------------------------------------------                                     ------------------------Co-Advisor’s Name                                                                     Signature

ABSTRACT

Load forecasting is a way of estimating what future electric load will be for a given forecast horizon based on the available information. Peak load forecasting is the procedure to predict future peak loading conditions.

This thesis work is a study about Ethiopian peak load forecasting for the next 10 years. In this thesis Hybrid method used for the peak load forecasting, the method is the combination of two models: (i) econometric model, which uses the multiple regression techniques   and (ii) end use model which use the data directly from the user demand.

The econometric approach combines economic theory and statistical techniques for forecasting. The approach estimates the relationship between peak load (dependent variables) and load influencing factor. In this thesis the economic and demographic factors are considered. In other word Gross Domestic Product (GDP) of Ethiopia and population of Ethiopia are the selected factors. The econometric approach combines economic theory and statistical techniques for forecasting electricity demand. In the econometric model multiple regression technique is used for mathematical analysis. This analysis is further compared with ARMAX mathematical model. A raw data regarding load demand covering previous period of five years data is used for this study. In this work peak load is used. The GDP and population of previous data with load data are used to get coefficients .The Mean Forecasting Error (MFE) also calculated to reduce the error. Using this coefficients and error in the mathematical equations, the next ten years forecast result is found.  The study also utilized c-sharp software and forecasted the Ethiopian ten years peak load demand and compared with the econometric method load forecasted data.

End-use model focuses on the various uses of electricity in the users. These models are based on the principle that electricity demand is derived from customer demand. Thus end-use model explains   load demand as a function of the number of users.

I

In this thesis the newly commenced load consumed sector is independently analyzed and data collected.

The end-use modeling, econometric modeling, and their combinations are the most often used methods for long-term load forecasting. Long-term forecasts include the forecasts on the population changes, economic development, industrial construction, and technology development. Here finally this thesis combines the two models and the 10 years forecasted result found by adding the models result.

II

ACKNOWLEDGEMENT

I thank God for leading me all the way through my life. I made it through my education and completed this thesis because of his help.

I would like to thank my advisor Dr.Soosai Mary Raja Slochanal for her support and guidance throughout my thesis. My sincere thanks to PG coordinator

Mr. Berhane who is  instrumental to guide me for the development of the thesis.

Also I thank all the persons who helped me to complete this work either directly or indirectly.

Finally I would like to thank my family for their support throughout this thesis.

III

TABLE OF CONTENTS

Abstract ………………….………..…………….…………………………………………….………………..I

Acknowledgement ……….……………….………………………………………………………..…… III

Table of contents .……………….……….……………….……………………………………………… IV

List of figures ..………………….……………………………………………………………………...…. VII

List of Table .. ……..….……………………………………….………………………………..…………..VIII

Abbreviation  …………..…………..…………………………….………………………………………… IX

Chapter one

1. Introduction  …………………….…………………………………………………………………………1

1.1 Background  …………………………………………………………………………………….……… 1

1.1.1 The energy sector in Ethiopia................................................................2

1.2 Objectives ………………………………………………….…………………..……………………….. 5

1.4.1 General Objectives………..…………….………………………..………………..5

1.4.2 Specific objectives..………………..……..………………………………..…….5

1.3 Statement of problem ………………………..……….….…………………….…….…………….5

1.4 Literature review   ……..…………………………………………………………….………………..7

1.5 Proposed method……..…….……………………………………………………….…………………9

IV

Chapter two

2. Overview of forecasting……………………………………………………………………………10

2.1 Econometric model …………………………………….………………………………………..10

2.2 Stochastic time series ……………………………………………………………………………10

2.3 Autoregressive (AR) model ………………………………………….…………………………11

2.3.1 Autoregressive moving-average (ARMA) model……………………………11

2.4 Regression……………………………………………………………………………….…..…………11

2.4.1 The linear regression model…………………………………………………..……..12

2.4.2 The multiple regression……………………………..…………………………………14

2.5  End-use models…………………………………………………………………………………..…15

2.6  Hybrid Econometric and End-Use Method…………………………………………..…15

Chapter Three

3. Problem formulation…………………………………………………………………………………17

3.1 Population Size ……………………………………………………………..…………….…………17

3.2  GDP in Ethiopia………………………………………………………………………………………18

3.3  Forecasting by multiple regression method……………………………………………19

3.4 Stochastic approach…………………………………………..……………………………………22

3.4.1 The ARMA model……………………………………………………….…………………………22

3.4.2 The ARMAX model………………………………………………..………………………………23

3.5  End-use models………….………………………..…………….…..………………………………27

V

3.5.1Transport Sector……………………………….…………….…..………………………………28

3.5.2. Agricultural Sector developments……………………..….……………………………29

3.5.3 New Industrial development………………………………….……………………………30

3.5.4   Large scale Dwelling house expansion Programs: ………..……………………30

3.5.5 Electricity export consumers; ………………………………………..……………………31

3.5.6 Summery of load demand by new power demand ……….……………………32

3.6 Hybrid Econometric and End-Use Method………………………..……………………33

Chapter 4

4. Simulation………………………………………………………………….………..……………………36

4.1 C-sharp simulation package details……………………………………………………..36

4.2Algorithm……………………………………………………………..………………………………36

4.3 C Sharp code…………………………………………………………………..………………………38

Chapter 5

5. Result and Discussion……………………………………….……………………………………….44

5.1 Econometric model result…………………………………….……..……………………..44

5.2 End use method…………………………………………………...…………………………….45

5.3 Hybrid model………………………………………………………..…………………………….46

6. Conclusion and recommendation……………………………………….………………………48

6.1 conclusion…………………………………………………………………………………………..48

6.2 Recommendation…………………………………………………..…………………………….48

REFERENCES…………………………………………………………………..……………………………….50

VI

LIST OF FIGURES

Figure 3.1: Comparison of the ARMAX and Multiple regression Method Forecasting results………………………………………………………………….…………..……………27

Figure 3.2: Peak Load demand by end user method……………………….……....…..….33

Figure 3.3: Graphical representation of the forecasting results…….……….………..35

Figure 4.1: Flow chart of the econometric model algorithm…….…………..……….…37

Figure 4.2: Ethiopian load forecast software front window…………….…..……….….38

Figure 4.3: Historical data………………..……………………………………………….…………...…39

Figure 4.4: Forecast window………………………………………….……………………..……….…40

Figure 4.5: Forecast values……………………………………………….…………………..……….…41

Figure 4.6: Forecasting for any given year…………………………….…………..………..……43

Figure 4.7: Change header window……………………………………….……………..……….…43

Figure 5.1: Graphical comparison of forecasting result………………………..……….…45

Figure 5.2: Comparison of result with EEP forecasted data…….…….……..……….…47

VII

LIST OF TABLES

Table 3.1 Population of Ethiopia forecasting....................................................... 17

Table 3.2 GDP of Ethiopia.................................................................................... 18

Table 3.3 Ethiopia Historical load data................................................................ 19

Table 3.4   Forecasting result of the multiple regression techniques.................. 22

Table 3.5: Load forecast result by ARMAX method…………………………………………….25

Table 3.6: comparison of ARMAX and multiple regression techniques forecasting result………………….…………………………………………………………………………...…………………26

Table 3.7  Ethiopian new rail way routes……..…………………………….………………………28

Table 3.8   Transport sector peak load demand………………………….…….….…………….28

Table 3.9 IAIP peak load ……………….…………………………………………………….………….…29

Table 3.10:  Industrial park peak load demand………………………………………….……….30

Table 3.11:  Peak load demand by AAHPO………………………………….……….…..………..31

Table 3.12:  Peak load demand by the export consumers………….………….…………..31

Table 3.13:  Summery result by end use model…………………………….…….…………….32

Table 3.14:   Ethiopian peak load forecasting result……………………..…….…………….34

Table 4.1:   Forecasting result using the C sharp software………………….…………….34

Table 5.1:   Comparison of forecasted result by econometric model…………………………………………………………………………………….……….….…………….34

Table 5.2:   Comparison of forecasted data…………………………..……..……….………….34

VIII

ABREVATION AND ACRONYMS

ARMA - autoregressive moving average

ARIMA - autoregressive integrated moving average

ARMAX - autoregressive moving average with exogenous variables

ARIMAX - autoregressive integrated moving average with exogenous variables

GDP - Gross Domestic product

EEU - Ethiopian electric utility

EEP - Ethiopian Electric power

AAHPO - Addis Ababa housing project office

IAIP – Integrated Agro Industrial Parks

GOE - Government of Ethiopia

ICS- Inter connected system

SCS – Self connected system

IX